| Advanced Industrial Design in Acoustic s.r.l. Spin-off dell’Università degli Studi di Parma |  |  |  | REPUBBLICA ITALIANA UNIVERSITÀ DEGLI STUDI DI PARMA |
| --- | --- | --- | --- | --- |
| Sede legale: v. Sicuri 60/A – 43124 PARMA Tel. 0521 969036 Fax. 0521 256963 C.F. – P.I. – R.I. 02285590341 www.aidasrl.it info@aidasrl.it |  |  |  |  |
| Sound Emission Level at the workplace of a GD DF10 machine |  |  |  |  |
| Data: |  | 19 Juky 2012 |  |  |
| Committente: |  | G.D. S.p.a. Via Battidarno, 91 - 40133 Bologna P.IVA 02075320370 |  |  |
| Oggetto: |  | Comparative analysis of emission sound level measurements performed on a DF10 machine (plus accessories) at GD Factory, (Bologna, ITALY) and at the production factory (Izhora Sankt Petersburg, Russian Federation) |  |  |
| Tecnici: |  | Prof. Ing. Angelo Farina Professore Ordinario – Università di Parma Tecnico Competente in acustica ai sensi della legge n.447/1995 farina@aidasrl.it |  |  |

INDICE

1	Introduction	3

2	Measurements at GD premises	5

3	Measurements at Philip Morris Izhora premises	9

4	Conclusions	14

5	References	15

- Introduction

This document contains an analysis of the results of two sets of measurements performed on the same machinery:

- Measurements performed at the GD Factrory by the GD Noise - Vibrations & Safety Test Lab. after finishing the assembly of the machine and of its ancillary equipment.

- Measurements at the customer’s workshop (Izhora Sankt Petersburg, Russian Federation) by the LABORATORY OF ERGONOMY AND ENVIRONMENTAL RESEARCH, Department of Mechanics &Vibroacoustics, AGH-UST Krakow

The results of the two sets of measurements differ by a few decibels, and this difference causes a completely different evaluation of the contractual clause regarding noise emission.

It is therefore very important to first describe this contractual clause, because in the end the very point of this document is to re-evaluate the measurement results, so that the cause of the differences is understood, and the evaluation of the respect of the clause is accomplished.

The contractual text is the following:

4.2.7. Noise Emission Level

The equipment must comply with the PMI requirements relating to noise emission sound pressure

levels. The calculated LpA values must be equal or less than 82 dB(A) in all workplaces.

The PMI requirements are:

- The supplier must carry out noise emission measurements at his premises according to ISO 11204 and issue a report based on this standard. Where it is impossible or impractical to carry out the measurements at the supplier’s premises, the measurements shall be carried out 'in situ' at the PMI factory.

  - At the suppliers’ premises, each machine must be measured individually. If several machines are connected to each other, the machines must be measured individually if possible.

  - At PMI factory the machines cannot be measured individually, so must be measured linked to each other. Some measurement points for the individual machine may therefore be invalid in situ.

- The measurement points must be submitted to PMIE for approval.

- The measurements must be taken at nominal speed and the equipment must be operated with materials.

- A-weighted time averaged emission sound pressure levels (LpA) must be calculated around the machine. Measurement points must be located at 1.0 m distance from the machine, at 1.6m height.

- The report must contain full details of measured sound pressure levels, room characteristics and the derived correction values

The contractual text above makes it clear that the PRIMARY way of performing the measurements is at supplier’s premises, and that these measurements should be carried out by the supplier (GD). This was done, and corresponds to the three measurements reports provided by GD and named:

- DF10_10000_MIUM.doc

- T10_10000_MIUM.doc

- DF10 PM Izhora 1.doc

The latter refers to a set of measurement conducted on the assembly constituted by the DF10 machine, 2 T10 machines, and other ancillary equipment, constituting the whole production unit.

Nevertheless, the contract is clear in stating that the relevant sound emission levels are those measured individually on each machine, and that the “cumulative effect” given by the simultaneous operation of more than one machine should NOT be taken into account.

So, the results of the measurements performed at the GD premises on the individual machines are those relevant for checking the compliance with the contractual value. The subsequent measurements performed at the customer’s premises are meant to be just a verification, and not hold the same contractual value, because the “cumulative effect” of operating simultaneously more than one machine cannot be avoided, as clearly admitted in the contractual clause.

The second point to be discussed here is the environmental effect. As the contractual clause refers explicitly to Sound Emission values, measured according to ISO 11024 (that is with accurate environmental correction), it is assumed that the measured value are completely unaffected by the reverberation or reflections due to the building in which the machine under test is located.

The strict conditions required for applying ISO 11024 are met at the supplier facilities, but of course it had been impossible to meet the same requirements in the production factory at the customer’s premises; of consequence, the measurements conducted by the University of Krakow were done according to ISO 11202, with approximate environmental corrections.

It is well known how the acoustical conditions usually existing in this type of factories cause a systematic underestimation of the environmental correction, if performed with the simplified methods described in ISO 11202. This is the main reasons for which the specific European standard EN 415-9:2009 (Safety of packaging machines. Noise measurement methods for packaging machines, packaging lines and associated equipment, grade of accuracy 2 and 3) was developed.

This standard describes detailed procedures for ensuring that the true value of the environmental correction factor is applied when measuring sound emission values of this type of machinery.

Of course, the usage of the EN standard is mandatory within the EC, but not in Russia. However, it would have been more appropriate to use this standard also in this case, as it is fully compliant with ISO 11204 (the contractual standard), and it just provides a more accurate way of evaluating the environmental effect than the approximate methods described in ISO 11202. Therefore, it is highly probable that the measurements performed according with ISO 11202 are causing a systematic underestimation of the environmental correction factor, and hence provide overestimated sound emission values.

The final point to be anticipated in this introduction is the position of the points where the compliance with the contractual value of 82 dB(A) must be ensured. The contractual text is very clear, the value of 82 dB(A) must not be exceeded “in all workplaces”, nowhere in the contractual text it is written that the value of 82 dB(A) must not be exceed “in all the measurement points”.

It must be made clear that, whatever standard is adopted (the thing being substantially the same in all of three, ISO 11202, ISO 11204 and EN 415-9), for evaluating correctly the environmental correction factor to be applied at the values measured at the workplace(s), a much larger number of measurement points is required, surrounding more or less completely the machine, with the goal of evaluating an “average level” to be compared with the level measured at the workplace. This difference will affect the calculation of the environmental correction factor to be applied at the measurements performed at the workplace, but of course all these additional measurement points, albeit being required for ascertain the correct value of the environmental correction factor, do not constitute “verification points” where the contractual maximum sound emission level must be met.

In reality, in a properly designed machine, it is usual to find that the workplace exhibit a local sound emission value which is significantly LOWER than the values measured in other positions around the machine, which are not a workplace. The correct design is aimed to protect the workplace: so it cannot be claimed to be a violation of the contractual cause about noise emission if the sound pressure level is found to be exceeding 82 dB(A) in measurement positions which are not the “official workplaces” of each machine, as clearly defined in the operator’s manual of each machine.

In the following chapters, each of the above points will be expanded, analyzing the measurement results, computing analytically the effect due to the simultaneous operation of several machines, re-working the procedure for assessing the environmental correction factor, and evaluating the degree of congruence of the measured values at the two locations, and their compliance with the contractual limit.

- Measurements at GD premises

The first set of data was measured at the GD premises, both on the individual machines and on the production unit constituted by one DF10 machine, two T10 machines and other equipment.

The following figures show the measurement points in these three cases:

Fig. 1 – measurement points around the DF10 machine performed at the GD premises

Fig. 2 – measurement points around the T10 machine performed at the GD premises

Fig. 3 – measurement points around the whole production unit (one DF10 + 2 T10 machines) performed at the GD premises

In the figures above, the workplaces of each machine, as declared in its operator’s manual, are marked with a red circle.

The following tables contain the sound levels measured in each of the three cases above:

Table I – measurement results at GD premises - single DF10 machine – EN415-9

| Point N° | L’pAi dB(A) | L’’pAi dB(A) | K1A dB(A) | K3A dB(A) | LpAi dB(A) |
| --- | --- | --- | --- | --- | --- |
| 1 | 78.0 | 50.0 | 0.0 | 3.0 | 75.0 |
| 2 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 3 | 78.5 | 50.0 | 0.0 | 3.0 | 75.5 |
| 4 | 80.0 | 50.0 | 0.0 | 3.0 | 77.0 |
| 5 | 80.0 | 50.0 | 0.0 | 3.0 | 77.0 |
| 6 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 7 | 77.0 | 50.0 | 0.0 | 3.0 | 74.0 |
| 8 | 76.0 | 50.0 | 0.0 | 3.0 | 73.0 |
| 9 | 77.0 | 50.0 | 0.0 | 3.0 | 74.0 |
| 10 | 77.0 | 50.0 | 0.0 | 3.0 | 74.0 |
| 11 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 12 | 77.0 | 50.0 | 0.0 | 3.0 | 74.0 |
| 13 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 14 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 15 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 16 | 78.5 | 50.0 | 0.0 | 3.0 | 75.5 |
| 17 | 79.5 | 50.0 | 0.0 | 3.0 | 76.5 |
| 18 | 79.5 | 50.0 | 0.0 | 3.0 | 76.5 |
| 19 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 20 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| 21 | 77.5 | 50.0 | 0.0 | 3.0 | 74.5 |
| Avg dB(A) | 78.0 | 50.0 | 0.0 | 3.0 | 75.0 |

Table II – measurement results at GD premises - single T10 machine – EN415-9

| Point N° | L’pAi dB(A) | L’’pAi dB(A) | K1A dB(A) | K3A dB(A) | LpAi dB(A) |
| --- | --- | --- | --- | --- | --- |
| 1 | 66.0 | 50.0 | 0.0 | 3.0 | 63.0 |
| 2 | 68.0 | 50.0 | 0.0 | 3.0 | 65.0 |
| 3 | 66.0 | 50.0 | 0.0 | 3.0 | 63.0 |
| 4 | 66.0 | 50.0 | 0.0 | 3.0 | 63.0 |
| 5 | 67.0 | 50.0 | 0.0 | 3.0 | 64.0 |
| 6 | 66.0 | 50.0 | 0.0 | 3.0 | 63.0 |
| Avg dB(A) | 66.5 | 50.0 | 0.0 | 3.0 | 63.5 |

Table III – measurement results at GD premises – CF10 machine plus 2 T10 machines

| POS. | dB(A)Leq. | POS. | dB(A)Leq. | POS. | dB(A)Leq. |
| --- | --- | --- | --- | --- | --- |
| 1 | 78,0 | 11 | 77,0 | 21 | 77,0 |
| 2 | 76,0 | 12 | 77,0 |  |  |
| 3 | 78,0 | 13 | 77,0 |  |  |
| 4 | 80,0 | 14 | 77,0 |  |  |
| 5 | 80,0 | 15 | 76,0 |  |  |
| 6 | 77,0 | 16 | 77,0 |  |  |
| 7 | 76,0 | 17 | 79,0 |  |  |
| 8 | 76,0 | 18 | 79,0 |  |  |
| 9 | 76,0 | 19 | 77,0 |  |  |
| 10 | 76,0 | 20 | 77,0 |  |  |

The values reported in the tables above require some comments:

- The “local” environmental correction factor, K3A, is in reality a constant value, the same in every point. This is due to the fact that this value is computed according to annex B.2 of EN 415:2009

- K1A is 0.0 dB everywhere, as the background noise in the workshop was negligible

- In the case of the whole production unit the values reported are IMMISSION values, not EMISSION values. Thus, no environmental correction has been applied. Hence, these values are just reported as indicative, but these values are NOT to be compared with the contractual EMISSION values. This goal is instead accomplished by the values marked with the yellow background in tables I and II.

As shown in the tables above, the average local environmental correction was approximately 3 dB(A). EN 415-9 allows for employing a constant value of the environmental correction factor as the local environmental correction factor at all the workplaces.

Of course, in case instead a strict application of ISO11204 is performed, at every measurement point a local value of the environmental correction factor, K3A, has to be applied.

The following tables provide such computations from the measurements performed on the DF10 and T10 machines.

Table IV – measurement results at GD premises - single DF10 machine – ISO 11204

| T30 (dBA) | 1.12 | s | According to ISO 3382 |
| --- | --- | --- | --- |
| Volume = | 4125 | m3 | Dimensions (m): 55x15x5 |
| A = | 588 | m2 | Absorption Area (Sabine) |
| Sm = | 105.56 | m2 | Measurement Surface (m): 8.2x3.8x3.1 |

| Point | L’pAi | L’’pAi | K1A | K3A | LpAi |
| --- | --- | --- | --- | --- | --- |
| N° | dB(A) | dB(A) | dB(A) | dB(A) | dB(A) |
| 1 | 78 | 50 | 0 | 2.4 | 75.6 |
| 2 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 3 | 78.5 | 50 | 0 | 2.0 | 76.5 |
| 4 | 80 | 50 | 0 | 1.3 | 78.7 |
| 5 | 80 | 50 | 0 | 1.3 | 78.7 |
| 6 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 7 | 77 | 50 | 0 | 3.3 | 73.7 |
| 8 | 76 | 50 | 0 | 4.7 | 71.3 |
| 9 | 77 | 50 | 0 | 3.3 | 73.7 |
| 10 | 77 | 50 | 0 | 3.3 | 73.7 |
| 11 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 12 | 77 | 50 | 0 | 3.3 | 73.7 |
| 13 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 14 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 15 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 16 | 78.5 | 50 | 0 | 2.0 | 76.5 |
| 17 | 79.5 | 50 | 0 | 1.5 | 78.0 |
| 18 | 79.5 | 50 | 0 | 1.5 | 78.0 |
| 19 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 20 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| 21 | 77.5 | 50 | 0 | 2.8 | 74.7 |
| Avg | 78.0 | 50 | 0 | 2.6 | 75.7 |
| dB(A) |  |  |  |  |  |

Table V – measurement results at GD premises - single T10 machine – ISO 11204

| T30 (dBA) | 1.12 | s | According to ISO 3382 |
| --- | --- | --- | --- |
| Volume = | 4125 | m3 | Dimensions (m): 55x15x5 |
| A = | 588 | m2 | Absorption Area (Sabine) |
| Sm = | 67.12 | m2 | Measurement Surface (m): 4.8x4.4x2.5 |

| Point | L’pAi | L’’pAi | K1A | K3A | LpAi |
| --- | --- | --- | --- | --- | --- |
| N° | dB(A) | dB(A) | dB(A) | dB(A) | dB(A) |
| 1 | 66 | 50 | 0 | 1.9 | 64.1 |
| 2 | 68 | 50 | 0 | 1.1 | 66.9 |
| 3 | 66 | 50 | 0 | 1.9 | 64.1 |
| 4 | 66 | 50 | 0 | 1.9 | 64.1 |
| 5 | 67 | 50 | 0 | 1.5 | 65.5 |
| 6 | 66 | 50 | 0 | 1.9 | 64.1 |
| Avg | 66.6 | 50 | 0 | 1.7 | 64.9 |
| dB(A) |  |  |  |  |  |

Comparing the values of K3A contained in tables IV and V with those contained in tables I and II, it is clear how the usage of the “accurate” local environmental correction factor K3A, computed according to ISO 11204, results in values of K3A which, at the workplaces, are significantly smaller than those contained in the original GD reports.

This is due to three factors:

- The workshop where the measurements were performed is an highly damped room, equipped with sound-proofed ceiling, which provides a low reverberation time and a large value of room’s absorption area A.

- On the DF10 machine, the workplace positions are those with maximum local sound pressure level L’pAi: and the algorithm contained in ISO 11204 assumes that the local variations in the values of L’pAi are due uniquely to the directivity of the sound source, and not to spatial disuniformity of the reverberant sound field. Hence, paradoxically, the computer local environmental correction factor K3A is smaller at positions where the measured sound pressure level L’pAi is larger.

- On the T10 machine, the same environmental correction factor of the DF10 machine was employed in the GD report, whilst the measurement surface Sm is much smaller.

However, despite the smaller values of K3A, the resulting emission levels at all the workplace positions are still significantly smaller than the contractual limit of 82 dB(A). The worst positions are n. 4 and 5 on the DF10 machine, with a value of 78.7 dB(A), which is 3.3 dB(A) smaller than the limit.

- Measurements at Philip Morris Izhora premises

The second set of data was measured at the Philip Morris Izhora premises on the complete production unit constituted by one DF10 machine, two T10 machines and other equipment.

Fig. 4 – measurement points around the whole production unit (one DF10 + 2 T10 machines + “Nilfisk cfm”) performed at the Philip Morris Izhora premises

This production unit corresponds almost exactly to the third set of measurements performed at GD Italy, but the number and position of the measurement points is not exactly the same.

However, it is expected that, after removing the background noise and the environmental effects, the results should be comparable with the measurements results reported here in table III (it must be remembered, indeed, that the values reported in table III are immission values, as the environmental correction factor K3A has not been subtracted).

The measurement surface Sm, along which the microphone positions have been located, has not been declared in the report of the University of Krakow. However, the total dimensions of the device under test have been declared as length=11.66m, width=6.93 m. Hence the measurement surface, located at 1.0 m distance form the machine, can be approximated to be equal to 13.66x8.93x3.20 m. And the corresponding measurement surface Sm becomes equal to 266.56 m2.

The room where the production unit was installed has dimensions of 72x44x8m, with a total volume V=25344 m3, and a total surface Sv=8192 m2, which is much larger than the workshop where the initial measurements were performed.

However, this just means that the room is much wider, bit still the height is quite limited. This is a typical “wide and short” building, which violates the geometrical properties required for the validity of the Sabine’s formula. In this type of buildings, it is wrong to employ the Sabine’s formula for deriving the value of the total absorption area A of the room, to be employed for computing the environmental correction factor K3A.

EN 415/9 provides guidance about the correct evaluation of the environmental correction factor in these buildings (which are typical of packaging industry).

However, unfortunately, the report of the University of Krakow does not contain all the information required for performing such accurate estimation. In particular, it appears that the direct assessment of the environmental correction factor by means of a reference sound source has not been performed. That the Sound Power measurement of the production unit by means of the Sound Intensity technique (ISO 9614-2) has net been done. And that the characterization of the room properties by the measurement of the spatial decay curve according to ISO 14257 has not been performed, too.

Even the very simple measurement of the room’s reverberation  time, which could have been useful for estimating the room absorption by some of existing not-Sabinian formulas, has been neglected.

It appears that the estimation of the room’s environmental effect K2A has been done according to ISO 3746, employing simply an estimated absorption coefficient  =0.25 and the total surface of the room SV:

A =   SV = 0.25  8192 = 2048 m2

Hence the environmental correction factor K2A becomes:

K2A=10⋅log1+4⋅SmA = 1.8 dB(A)

Which is the value shown (without any analytical justification) in the report of the University of Krakow.

As discussed above, this approach is certainly causing a significant error, because this room does not comply with the geometrical constraints for the Sabine formulation to be valid. Furthermore, the “choice” of the value of the average absorption coefficient =0.25 appears to be over optimistic.

The following table, taken by ISO 3746, is a sort of “guide” for selecting the appropriate value of the absorption coefficient :

Not having been there, and without any measurement results capable of characterizing, directly or indirectly, the absorption of the room, it is not easy to select the proper value from the table above. However, from the few verbal information collected by the undersigned, the most appropriate choice could be the second one, =0.1.

With this choice, we get:

A =   SV = 0.1  8192 = 819.2 m2

Hence the environmental correction factor K2A becomes:

K2A=10⋅log1+4⋅SmA = 3.62 dB(A)

Which appears a much more realistic value, considering a not-sound-proofed building and a large measurement surface. This is with every possibility still a strongly underestimated vale, as it is common, with equipment of this size, and in this type of rooms, to find values of K2A greater than 5 dB(A). However this already show the point, that ism, the environmental effect has been drastically underestimated in the report of the University of Krakow.

The following table shows the results as they appear in their report:

Table VI – measurement results of the University of Krakow at IZHORA, Sankt Peterburg

The workplace positions, where the contractual values should be met, are marked with red rectangles. It appears that the contractual values are NOT met at positions n. 4 and 5 (that is, at the operator’s position of the DF10 machine).

However, as already anticipated, the computation of the emission values was done with underestimated environmental correction factor, and the “cumulative effect” due to the proximity of another machine (the T10) must be taken into account, both in term of noise emission, and of local reflection.

IT is not possible here to provide an accurate evaluation of the latter effects. However, it is easy to recomputed the table above, applying a more realistic environmental correction factor K3A, based on a value of K2A equal to 3.62 dB(A) instead of 1.8 dB(A). The following table contains such a computation.

Table VI – measurement results at PM premises (IZHORA) – ISO 11202

| Point | L’pAi | L’’pAi | K1A | K2A | K3A | LpAi |
| --- | --- | --- | --- | --- | --- | --- |
| N° | dB(A) | dB(A) | dB(A) | dB(A) | dB(A) | dB(A) |
| 1 | 80.2 | 74.0 | 1.2 | 3.62 | 3.0 | 76.0 |
| 2 | 79.6 | 72.9 | 1.0 | 3.62 | 3.7 | 74.8 |
| 3 | 82.1 | 73.8 | 0.7 | 3.62 | 1.7 | 79.7 |
| 4 | 83.9 | 74.0 | 0.5 | 3.62 | 1.0 | 82.4 |
| 5 | 83.6 | 74.0 | 0.5 | 3.62 | 1.1 | 82.0 |
| 6 | 82.2 | 73.6 | 0.6 | 3.62 | 1.6 | 79.9 |
| 7 | 80.6 | 73.0 | 0.8 | 3.62 | 2.6 | 77.1 |
| 8 | 78.7 | 72.9 | 1.3 | 3.62 | 5.3 | 72.0 |
| 9 | 78.1 | 72.6 | 1.4 | 3.62 | 7.0 | 69.7 |
| 10 | 77.3 | 72.9 | 2.0 | 3.62 | 7.0 | 68.3 |
| 11 | 77.3 | 72.5 | 1.7 | 3.62 | 7.0 | 68.6 |
| 12 | 77.2 | 72.7 | 1.9 | 3.62 | 7.0 | 68.3 |
| 13 | 77.2 | 72.5 | 1.8 | 3.62 | 7.0 | 68.4 |
| 14 | 77.5 | 73.0 | 1.9 | 3.62 | 7.0 | 68.6 |
| 15 | 77.8 | 73.5 | 2.0 | 3.62 | 7.0 | 68.8 |
| 16 | 77.9 | 74.1 | 2.3 | 3.62 | 7.0 | 68.6 |
| 17 | 78.5 | 74.4 | 2.1 | 3.62 | 5.9 | 70.5 |
| 18 | 79.7 | 74.9 | 1.7 | 3.62 | 3.6 | 74.4 |
| 19 | 79.3 | 75.0 | 2.0 | 3.62 | 4.2 | 73.1 |
| 20 | 78.5 | 74.9 | 2.5 | 3.62 | 5.9 | 70.1 |
| 21 | 79.1 | 74.8 | 2.0 | 3.62 | 4.5 | 72.6 |
| 22 | 78.7 | 74.8 | 2.3 | 3.62 | 5.3 | 71.1 |
| 23 | 78.5 | 73.8 | 1.8 | 3.62 | 5.9 | 70.8 |
| 24 | 77.9 | 74.0 | 2.3 | 3.62 | 7.0 | 68.6 |
| 25 | 77.7 | 73.3 | 2.0 | 3.62 | 7.0 | 68.7 |
| 26 | 77.3 | 73.4 | 2.3 | 3.62 | 7.0 | 68.0 |
| 27 | 78.7 | 73.6 | 1.6 | 3.62 | 5.3 | 71.8 |
| 29 | 80.0 | 74.3 | 1.4 | 3.62 | 3.2 | 75.4 |
| 30 | 80.1 | 75.2 | 1.7 | 3.62 | 3.1 | 75.3 |
| 31 | 79.9 | 74.0 | 1.3 | 3.62 | 3.3 | 75.3 |
| 32 | 79.2 | 73.2 | 1.3 | 3.62 | 4.3 | 73.6 |
| 33 | 80.0 | 73.6 | 1.1 | 3.62 | 3.2 | 75.6 |
| 34 | 79.9 | 73.7 | 1.2 | 3.62 | 3.3 | 75.4 |
| 35 | 80.1 | 73.7 | 1.1 | 3.62 | 3.1 | 75.9 |
| 36 | 80.1 | 72.8 | 0.9 | 3.62 | 3.1 | 76.1 |
| 37 | 79.4 | 73.5 | 1.3 | 3.62 | 4.0 | 74.1 |
| 38 | 79.3 | 73.9 | 1.5 | 3.62 | 4.2 | 73.7 |
| 39 | 79.9 | 73.4 | 1.1 | 3.62 | 3.3 | 75.5 |
| 40 | 80.9 | 73.4 | 0.9 | 3.62 | 2.4 | 77.6 |
| Avg | 79.7 | 50 | 1.5 | 3.6 | 4.6 | 75.2 |
| dB(A) |  |  |  |  |  |  |

Despite the much larger value of K2A, the values of K3A at points 4 and 5 are even smaller than what calculated by the University of Krakow. The reason is that the logic of ISO 11204 penalizes points having local values of the sound pressure level larger than the average value, and instead gives great reduction to points where the sound pressure level is already below average.

This is a known artefact of ISO 11204. Exactly for this reason, EN415-9 warns against employing such “local” values of the environmental correction factor when any of these facts becomes true:

- ⎯the machine has strong emission from its top, (e.g. machine which is completely surrounded by soundproof shields but with open area above);

- ⎯the emission from the top is negligible compared to the emission from localized points around the machine;

- ⎯the work station is not properly shielded from the noise radiated by the machine;

- ⎯the reverberant field is not uniform around the machine (e.g. the machine is close to a reflecting  surface).

In this case, clauses 3 and 4 appear to be true. Hence, it is simply wrong to correct the measured values employing the local environmental correction factor K3A. EN 415-9 suggests, in these cases, to employ everywhere, and particularly at the workplace, the “average” environmental correction factor K2A.

EN 415-9 also warns about employing the “Sabinian” formulation for evaluating K2A when these facts occur:

⎯Strong sound reflections from walls and ceiling;

⎯Uneven distribution of the absorption;

- ⎯Low and large room (w / h > 3, where: w = smaller lateral dimension and h= height);

- ⎯Very large room (V > 2500 m3, where V=volume of room).

In this case, the last two facts are certainly true. When this happens EN 415-9 suggest to proceed by evaluating directly K2A as follows:

In all these cases "in situ" experimental tests should be made to determine the environmental correction K2 (e.g. A.3, EN ISO 3744:1995, or A.3.1, EN ISO 3746:1995), employing a distance from the reference sound source and the measurement point equal to the value of “a” defined in Equation (B.3) above.

The only fact that the results of the measurements performed at the IZHORA factory are systematically divergent form those performed at GD Italy on the whole production unit are the proof that the environmental effect at IZHORA is much larger than what can be established according to ISO 11202 / ISO 11204. The only way to proof this, of course, is to perform a direct measurement of the environmental correction factor K2A in the IZHORA factory, employing any of the currently available methods.

The undersigned suggests to employ ISO 11203, after having performed an accurate measurement of the sound power level Lw by means of the Sound Intensity method (ISO 9614-2).

Alternatively, the environmental effect of the building can be assessed employing a reference sound source of known sound power level.

However from a contractual point of view, it is clear how the relevant measurements are those performed at GD Italy by the supplier, on the individual machines.

- Conclusions

The case discussed here is quite typical, and the undersigned has seen this happening dozens of times. A machine is guaranteed in terms of sound emission level, and it results perfectly compliant when measured inside a room with small environmental effect.

Then the machine is installed in the factory, in a “low and large” room. Here the measured sound pressure levels become much greater, and employing the “normal” measurement standards, designed for Sabinian rooms, the environmental correction which can be applied is small.

So the machinery appears to have emission values much larger than those measured in a properly-soundproofed room.

Of course, this cannot happen, as, by definition, the emission values should NOT depend on the room where the equipment is installed. What is wrong is the procedure for assessing the environmental correction factor in the factory, as the methods currently described both in ISO 11202 and in ISO 11204 are over-simplified, and inherently assume the Sabine pre-requisites to be true (perfectly diffuse sound field, local level variation only due to source’s directivity and not to spatial variation of the environmental effect).

This problem has been currently solved, within the EU, by the recent standard EN 415-9, which recognizes the above problems, and gives guidance for the proper assessment of the true environmental correction factor in this type of buildings.

It is opinion of the undersigned that the usage of this standard should be extended worldwide, as of course this problem does not occur just within the EU.

However, whenever the application of this standard is not allowed, the only way of proofing the anomalous environmental amplification effect, which occurs in these buildings, is by employing measurement methods which allow for the direct measurement of the environmental correction factor K2A (Sound Intensity, Reference Sound Source). Often this is the only way of proofing to the customer that the high sound pressure levels which are measured at the workplace are due to his acoustically-poor building, and not by excessive sound emission of the machinery.

The undersigned analysed the physical causes of these very large values of the environmental correction factors occurring in “low and large” rooms, and published several papers on this [1,2,3,4]. The result is that it is possible to evaluate the environmental effect inside a low and large room by means of either simplified analytical formulas (the Farina/Fornari formula) or by numerical simulations (provided that a simulation program capable of dealing with non-Sabinian volumes is employed).

This means that there is no need, in reality, to go to the factory and to perform complex measurements, as those required for direct assessment of the environmental correction factor, as  these analytical or numerical methods provide almost the same accuracy, at a fraction of the cost.

However, the suggestion in these cases is to first explain what’s happening to the customer: not only it is important to clarify the point related to meeting the contractual values for the emission values, but it very important to provide the costumer of information about what’s happening inside his building, about the safety risks to which he is exposing his employees, about his responsibility due to an improper set up of the working environment (which of course is NOT compliant with the recommendations given in ISO 11690, parts 1 and 3), and about the possibility to ameliorate significantly the situation by employing a suitable room soundproofing (which hence provides much greater attenuation, than what can be expected by the Sabine’s formulation).

It can be useful to provide the customer with the papers listed below, and with a specific Powerpoint presentation (in English) [5] which has been prepared for explaining in explicit terms these almost unknown acoustical effects.

- References

[1]	A. Farina, G. Fornari, “Studio della propagazione del rumore in ambienti industriali bassi e vasti”, Atti del convegno dBA-98, Modena, 16-18 settembre 1998

[2]	G. Fornari, A. Farina, “Studio della propagazione del rumore in ambienti industriali bassi e vasti”, Industria delle Bevande (ISSN 0390-0541), anno 27, n. 158, Dicembre 1998, pp. 585-596

[3]	A. Farina, G. Fornari, “Simulazione della propagazione sonora”, Il Progettista Industriale, ISSN 0392-4823, N. 7, Luglio 2000, pp. 48-52

[4]	A. Farina, G. Fornari, “Propagazione del rumore in ambienti industriali bassi e vasti	”, Analisi & Calcolo, anno IV, n. 12, Gen-Mar 2003, pp. 23-26

[5]	A. Farina – “Sound propagation in low and large factories” – Powerpoint presentation, 2005

Parma,  19 July 2012

|  | Prof. Ing. Angelo Farina Tecnico Competente ai sensi della legge n. 447 / 1995 |
| --- | --- |