Applied Acoustics - 25/10/2019			In-class test - Lecturer: Angelo Farina

Note: some input date are based on the 6 digits of Matricula number, assigned to the 6 letters A B C D E F.

If for example the matricula is 123456, it means that A=1, B=2, C=3, etc. .

Furthermore CD=34 (NOT 3x4), DE =45, EF =56.

Warning: On-line compilation of this form warrants TWO additional score points.

Top of Form

Surname and Name

F

E

D

C

B

A

Matricula

In free field the SPL reduces with the distance from a point source with a law of:	one answer only, mandatory

- 3 dB/ m

- 3 dB/ doubling distance

- 6 dB/ m

- 6 dB/ doubling distance

- It depends on the wind speed gradient and the temperature gradient

In free field the SPL reduces with the distance from a line source with a law of:		one answer only, mandatory

- 3 dB/ m

- 3 dB/ doubling distance

- 6 dB/ m

- 6 dB/ doubling distance

- It depends on the wind speed gradient and the temperature gradient

Select only the correct statements:							multiple answers allowed

- The human ears are basically sensors of the sound pressure

- The human ears are basically sensors of the particle velocity

- An omni microphone is a sensor of sound pressure

- An omni microphone is a sensor of particle velocity

- A cardioid microphone is a sensor of particle velocity

- A cardioid microphone is sensitive half to pressure, half to velocity

When calibrating a sound level meter using a standard calibrator what do you expect to measure?													multiple answers allowed

- A sound pressure having an RMS value of 1.0 Pa at 1 kHz

- A sound pressure level of 94 dB

- A sound pressure level of 94 dB(A)

- A sound pressure level of 94 dB(C)

- A particle velocity level of 94 dB

- A sound intensity level of 94 dB

A point source is located outdoors, over a reflecting plane. Measurements are made all around, at a distance of 10+F m, and the resulting average SPL is 80+E dB(A). Estimate the sound power level Lw of the sound source.

write number and measurement unit

The same source of previous exercise is placed inside a reverberant room, having a volume V=200+E*10 m³ a surface of 200+F*10 m² and an average absorption coefficient α = 0.2+D/100. Compute the average value of SPL inside the room, far from the source.

write number and measurement unit

If the source of previous exercise is placed on the reflecting floor inside the reverberant room, compute the SPL at the critical distance from the source.

write number and measurement unit

A machine produces an SPL of 90+F dB at 50+F*10 Hz. Compute the A-weighted SPL

write number and measurement unit

Scores: +1 for each correct answer, -1 for each wrong answer, 0 for no answer