Applied Acoustics - 15/11/2019			In-class test - Lecturer: Angelo Farina

Note: some input data are based on the 6 digits of Matricula number, assigned to the 6 letters A B C D E F.

If for example the matricula is 123456, it means that A=1, B=2, C=3, etc. .  Furthermore CD=34 (NOT 3x4).

Top of Form

Surname and Name

F

E

D

C

B

A

Matricula						Signature

1) What is the definition of  Modulation Transfer Function ?				(one answer only)

- The reduction of modulation due to noise and echoes

- The ratio between the initial modulation and the modulation of the received sound

- The ratio between modulation of the received sound and the initial modulation

- The signal to noise ratio for a given modulation frequency

- The value of STI for a given octave band

2) An exponential sine sweep is convolved with the time reversal of itself (instead of using its matched, amplitude-modulated inverse filter). What will be the spectral slope of the resulting impulse signal, as shown by an FFT analysis?									(one answer only)

- 0 dB/octave - flat (white)

- -3 dB/octave (pink)

- -6 dB/octave (brown)

- -9 dB/octave

- -12 dB/octave

3) What are the benefits of a clapping machine instead of other impulsive sources (such as balloons, pistols, firecracker, loudspeaker, etc.) ?								(multiple answers)

- Low cost

- Almost perfectly flat spectrum

- Highly reproducible

- Large acoustical power

- Omnidirectional

4) Select only the CORRECT facts							(multiple answers)

- Usually a good sound absorbing material is also a good sound insulating material

- A sound absorber is light, soft and porous, instead a sound insulating panel is heavy, rigid and airtight

- a, r and t are commonly employed coefficients, usually found on the data sheet of every material

- α is the coefficient usually declared by the manufacturer for a sound absorbing material

- In case of discrete objects, such as suspended baffles or seats, instead of providing the value of α, the manufacturer usually provides directly the value of A, in m².

5) In a noiseless classroom, the value of the MTF (Modulation Transfer Function) is equal to 0.5+F/20. Recompute the value of MTF when noise is added, with a Signal/Noise ratio of 5+E dB

(write number and measurement unit)

6) In a room, with a volume V=150+EF m³, and with an initial reverberation time of 3+E s, a total surface of 100+DE m² of sound absorbing panels is inserted, having an absorption coefficient α = 0.5+F/50. Compute the reverberant sound level reduction ΔL=L1-L2 caused by the increase of sound absorption.

(write number and measurement unit)

7) A vibrating panel is made of wood (having a density of 700+F*10 kg/m³), a thickness of 3+E/5 mm, and is mounted with a rear air gap of 50+D mm in front of a rigid wall. Compute the frequency of maximum absorption.

(write number and measurement unit)

8) A perforated rigid panel operates as a number of Helmoltz resonators. It has a thickness of 3+E/5 mm, and each squared meter carries a total of 100+F*20 holes, each with a diameter of 2+D/10 mm. The rear air gap is 50+D mm in front of a rigid wall. Compute the frequency of maximum absorption.

(write number and measurement unit)