Applied Acoustics – Test 3/2020			In-class test - Lecturer: Angelo Farina

Note: some input date are based on the 6 digits of Matricula number, assigned to the 6 letters A B C D E F.

If for example the matricula is 123456, it means that A=1, B=2, C=3, etc. .

Furthermore CD=34 (NOT 3x4), DE =45, EF =56.

If L =100+E*10 the results is 100+(5*10) = 150 (the product has precedence over the addition).

Top of Form

Surname and Name

F

E

D

C

B

A

Matricula

- What is the definition of the Sound Reduction Index R?

one answer only: 1 point if correct, -1 point if wrong, 0 point if "no answer"

- It is the reduction of SPL caused by a wall, that is the difference between the SPL at the two sides of it

- It is the "insertion loss", given by the difference of the SPL at the receiver before and after installing the wall

- It is given by -10*log10(t), where t is the transmission coefficient of the wall

- It is the ratio between the sound intensity passing through a wall and the sound intensity impinging over the wall

- It is the difference between the sound power level of the source and the sound pressure level behind the wall

- I do not know (no answer)

2) What is the effect of adding absorption to the receiving room on the value of the sound reduction index R of the partition?

one answer only: 1 point if correct, -1 point if wrong, 0 point if "no answer"

- R increases, as the absorption causes a reduction of the SPL in the receiving room

- R decreases, as in the formula for computing R the absorption area of the receiving room is at the denominator

- R does not change, as it only depends on the partition, and not on the acoustical properties of the two rooms

- R usually changes, but it is impossible to know if it will increase or decrease

- It depends on where the additional absorption is located: R increases if it is on the partition, R decreases if it is elsewhere

- I do not know (no answer)

3) Which of the following acoustical parameters can be used for qualifying the acoustical quality of a room used for conferences and lessons?

multiple answers allowed: for each answer, 1 point if correct, -1 point if wrong, 0 point if "not selected"

- Clarity C80

- Clarity C50

- IACC (inter-aural cross-correlation)

- Jlf (lateral fraction)

- STI

- Reverberation Time T20

4) Which of the following materials provide good sound absorption?

multiple answers allowed: for each answer, 1 point if correct, -1 point if wrong, 0 point if "not selected"

- closed-cell polystirene panels

- open-cell polyurethane foam

- polyesters fiber panels

- concrete slabs

- brick wall, plastered

- thick carpet

5) A plane wave having a sound intensity level of 80+F dB impinges over a large, planar surface having an absorption coefficient α=0.4+E/40. Compute the sound intensity level of the reflected plane wave.

write number and measurement unit

6) A plane wave having a sound intensity level of 80+F dB at the frequency of 400+D*100 Hz impinges over a wall made of concrete (ρ=2000+E*50 kg/m³), having a thickness of 10+F cm. Compute the sound intensity level of the plane wave which passes through the wall.

write number and measurement unit

7) In a laboratory for measuring the sound insulation of partitions, the following data are measured: L1 = 80+F dB (source room), L2 =30+E dB (receiving room). The partition has a surface S = 10+D/5 m², and the receiving room has an absorption area A = 5+E/2 m². Compute the value of R.

write number and measurement unit

8) A tapping machine is operated inside a multi-store building. In the receiving room the SPL is 60+F dB, and the reverberation time is 1+E/10 s. Compute the value of the normalized tapping level Ln according to the UNI 8270-4 standard.

write number and measurement unit