Applied Acoustics – Test 4/2020			In-class test - Lecturer: Angelo Farina

Note: some input date are based on the 6 digits of Matricula number, assigned to the 6 letters A B C D E F.

If for example the matricula is 123456, it means that A=1, B=2, C=3, etc. .

Furthermore CD=34 (NOT 3x4), DE =45, EF =56.

If L =100+E*10 the results is 100+(5*10) = 150 (the product has precedence over the addition).

Top of Form

Surname and Name

F

E

D

C

B

A

Matricula

- What is the definition of the Single Event Level SEL?

one answer only: 1 point if correct, -1 point if wrong, 0 point if "no answer"

- It is the max instantaneous SPL caused by the passage of a vehicle

- It is the average SPL during the time of passage of a vehicle

- It is the "dose" of acoustical energy associated with the passage of a vehicle

- It is the value measured during homologation of a vehicle and written on the document

- It's the A-weighted equivalent level measured during 1s, during which the SPL caused by the vehicle passby is maximum

- I do not know (no answer)

2) What is the definition of the Personal Exposure Level Lep?

one answer only: 1 point if correct, -1 point if wrong, 0 point if "no answer"

- It is the A-weighted equivalent level measured during an 8-h work day

- It is the A-weighted equivalent level measured during the duration of the work day

- It is the A-weighted equivalent level measured during the duration of the work day and rescaled to a nominal duration of 8h

- It is the SEL measured during the duration of the work day

- It is the SEL measured during the duration of the work day and rescaled from 1s to 8h

- I do not know (no answer)

3) Which methods can be used for measuring the absorption coefficient of a small sample of material, measuring just a few cm per side?

multiple answers allowed: for each answer, 1 point if correct, -1 point if wrong, 0 point if "not selected"

- ISO 354 in a reverberant room

- ISO 10534 Standing Wave Tube

- EN 1793-5 Impulsive Method

- Sound Intensity method in free field

- Sound Intensity method in a reverberant room

- Sound Intensity method inside a tube

4) Which of the following convolution algorithms provide higher performances on a modern computer?

multiple answers allowed: for each answer, 1 point if correct, -1 point if wrong, 0 point if "not selected"

- direct form convolution in time domain

- single-large-block (unpartitioned) overlap-and-save using FFT (Oppenheim-Shafer book)

- small-blocks (uniformly partitioning) overlap-and-save using FFT (Anders Torger, BRUTEFIR)

- variable-blocks (not-uniformly partitioned) overlap-and-save using FFT (Fons Adriaensen, J-conv, MCFX))

- hybrid method (Barry Kulp): a first block in direct form for zero-latency, followed by uniformly-partitioned blocks (Zoran DSP)

- hybrid method (Bill Gardner): a first block in direct form for zero-latency, followed by not-uniformly-partitioned blocks (Lake DSP; Dolby)

5) Compute Leq at the distance of 50+Fm from a road where each hour 500+E*30 cars are passing, each producing a SEL of 80+D dB(A) at the reference distance of 7.5 m.

write number and measurement unit

6) Compute Lep for a worker spending his (long) work day as follows: 4h at 75+F dB(A), 2h at 83+E/5 dB(A) and 4h at 79+D/4 dB(A).
write number and measurement unit

7) In a reverberant room having a volume of 200 + D*10 m3 the following data are measured: T1 (empty) = 6+F/4 s , T2(sample) = 2+E/5 s. The sample is made of 12 upholstered seats for a concert hall. Compute the equivalent absorption area A of one seat.

write number and measurement unit

8) A measurement is performed using the EN 1793-5 method in normal incidence. The microphone has a distance dsm from the source of 1+D/10 m, and a distance dm from the noise barrier of 0.3+E/5 m. The direct sound has an SPL=100 dB, and the reflected sound has an SPL of 80+F dB. Compute the absorption coefficient of the noise barrier.

write number and measurement unit