Applied Acoustics – Test 5/2020			In-class test - Lecturer: Angelo Farina

Note: some input date are based on the 6 digits of Matricula number, assigned to the 6 letters A B C D E F.

If for example the matricula is 123456, it means that A=1, B=2, C=3, etc. .

Furthermore CD=34 (NOT 3x4), DE =45, EF =56.

If L =100+E*10 the results is 100+(5*10) = 150 (the product has precedence over the addition).

Top of Form

Surname and Name

F

E

D

C

B

A

Matricula

- A generic microphone is partially sensitive to sound pressure, partially sensitive to particle velocity: which are the two percentages for a CARDIOID microphone?

one answer only: 1 point if correct, -1 point if wrong, 0 point if "no answer"

- 100% pressure, 0% velocity

- 75% pressure, 25% velocity

- 50% pressure, 50% velocity

- 25% pressure, 75% velocity

- 0% pressure, 100% velocity

- I do not know (no answer)

- What is the geometrical layout for an ORTF stereo microphone pair?

one answer only: 1 point if correct, -1 point if wrong, 0 point if "no answer"

- Two coincident cardioids, angled 60°

- Two coincident cardioids, angled 90°

- Two cardioids, spaced 170mm and angled 60°

- Two cardioids, spaced 170mm and angled 110°

- Two hypercardioids, spaced 300m and angled 90°

- I do not know (no answer)

3) How many channels are required for transporting an Ambisonics 7th-order stream?

one answer only: 1 point if correct, -1 point if wrong, 0 point if "no answer"

- 9 channels

- 16 channels

- 25 channels

- 34 channels

- 64 channels

- I do not know (no answer)

4) Which of the following playback systems provide periphonic reproduction (sound coming also from above and below)?

multiple answers allowed: for each answer, 1 point if correct, -1 point if wrong, 0 point if "not selected"

- 5.1 Dolby Digital Surround (DVD disc)

- 7.1 Surround (Blue Ray Disc)

- WFS (Wavefield Synthesis)

- Ambisonics

- SPS (Spatial PCM Sampling, for example Mach1)

- Dolby Atmos (sound objects)

5) Compute the number of channels required for an Ambisonics stream of order 5*F.

write number and measurement unit

6) Compute the beamwidth (angle from axis at which the level drops 3dB) for a virtual cardioid microphone of order 1+E.
write number and measurement unit

7) Two sound sources are separated by an angle of 60+F*3 degrees. We aim at them with two virtual cardioids of order 1+E. Compute the attenuation in dB of source B for the microphone aimed on source A.

write number and measurement unit

8) In a stereo mix, a highly-pitched sound is panned with an amplitude gain of (70+F)% on channel Left, and (30-F)% on channel Right. Assuming that the playback is occurring on a standard stereo system (with loudspeakers spaced 60° apart), find the apparent azimuth of the sound (counting from an azimuth=0° in the Center-Front).

write number and measurement unit